SECTION 07 62 10

EXTERIOR FLASHING RECEIVERS

PART 1 - GENERAL

1.01	SUMMARY

A.	This Section specifies a two-piece flashing receiver and clip, typically for attaching flashing at exterior wall openings.

B.	Related Sections:

1.	04 05 23	Masonry Accessories.

2.	06 16 00	Sheathing.

3.	07 27 00	Air Barriers.

4.	07 60 00	Flashing and Sheet Metal.

1.02	SUBMITTALS

A.	Product data:  Submit manufacturer's technical data sheet and details.

B.	Samples:  Full size sample, minimum 4 inches long.

1.03	QUALITY ASSURANCE

A.	Manufacturer Qualifications:  Minimum 5 years experience producing similar products.

PART 2 - PRODUCTS

2.01	FLASHING RECEIVERS

A.	Exterior Flashing Receivers:  AirSill™ by York Manufacturing, Inc. with the following characteristics:

1.	Receiver and Retainer Clip:  Extruded aluminum, 0.045 inch thick.

2.	Available Lengths:  5' and 10' sections with custom lengths available.

PART 3 - EXECUTION

3.01	INSTALLATION

A.	Exterior Flashing Receivers:  Install in accordance with manufacturer's recommendations, approved submittals and in proper relationship with adjacent construction.  Includes the following:

1.	Secure receiver at perimeter of wall opening with adhesives or fasteners.

2.	Place flashing into receiver channel.

3.	Secure flashing with receiver clip.

END OF SECTION 07 62 00